UR12e + Robotiq 2F-85

Real Workcell Startup & Operation Guide

ROS2 Humble  ·  MoveIt2  ·  v1.1.10

Branch: ur12e_2f-85_action_server

| Robot | Universal Robots UR12e (e-Series) |
| --- | --- |
| Gripper | Robotiq 2F-85 (via URCap + Action Server bridge) |
| Stack | ROS2 Humble · ros2_control · MoveIt2 · ur_robot_driver |
| Package | my_ur_description (workspace: /ros2_ws) |

⚠  SAFETY FIRST: Always keep the emergency stop (E-Stop) accessible. Test the E-Stop before every session. Clear the workspace before starting any motion. Operate at reduced speed (≤10%) during initial testing.

# 1  System Architecture

## 1.1  Control Mode Used (Real Workcell)

The real workcell uses Control Mode 4 — the production configuration that drives the physical UR12e arm and Robotiq 2F-85 gripper. The architecture avoids any competing action servers:

| Component | Role |
| --- | --- |
| ur_robot_driver | Connects to real UR12e via robot_ip; publishes joint states at 500 Hz |
| scaled_joint_trajectory_controller | Sends trajectory goals to the arm (speed-scaled by UR safety layer) |
| robotiq_gripper_controller | Always spawned — owns the follow_joint_trajectory action, publishes gripper joint states for RViz |
| robotiq_real_gripper_server.py | Subscribes to /robotiq_bridge/command (Float32). Sends URScript to UR tool port for real gripper motion. Mirrors the goal to ros2_control for RViz sync. |
| joint_state_broadcaster | Publishes /joint_states — feeds both RViz and your motion scripts |

## 1.2  Command Flow

Your script  ──►  publish Float32 to /robotiq_bridge/command  ──►  robotiq_real_gripper_server.py  ──►  URScript TCP socket to UR controller  ──►  Physical 2F-85 gripper moves  ──►  Mirror goal to ros2_control  ──►  RViz updates

For the arm, your script sends MoveGroup action goals to MoveIt, which plans a trajectory and sends it to scaled_joint_trajectory_controller, which drives the real UR12e joints.

## 1.3  Key Topics and Ports

| Topic / Port | Type | Purpose |
| --- | --- | --- |
| /robotiq_bridge/command | Float32 | Send gripper command (0.0=open, 0.8=closed rad) |
| /robotiq_bridge/position | Float32 | Read actual gripper position feedback |
| /joint_states | JointState | All joint positions — arm + gripper |
| move_action | MoveGroup | MoveIt2 motion planning interface |
| robot_ip:30002 | TCP | URScript port (used by gripper bridge) |
| robot_ip:29999 | TCP | UR Dashboard server |

# 2  Pre-Startup Checklist

Complete this checklist in order before every session. Do not skip any item.

## 2.1  Physical Hardware Checks

|  | Check | ✓ |
| --- | --- | --- |
| 1 | Robot is securely mounted — all base bolts tight, no play |  |
| 2 | 2F-85 gripper is firmly attached to tool flange — no wobble |  |
| 3 | Gripper cable routed safely — no tension, not crossing robot joints |  |
| 4 | Gripper fingers aligned and undamaged — visually inspect both fingers |  |
| 5 | Workspace is clear — no tools, objects, or people in reach envelope |  |
| 6 | E-Stop button is accessible from your operating position |  |
| 7 | Network cable between PC and UR12e controller is connected |  |
| 8 | UR controller power is ON — blue indicator light on teach pendant |  |

## 2.2  UR Teach Pendant Setup

- Power on the UR12e controller and wait for PolyScope to boot (~60 seconds).

- Clear any faults: tap the red fault banner → 'Restart Safety'. If a protective stop appears, resolve the cause first.

- Load the ExternalControl program: Program Tab → URCaps → External Control → set Host IP to your PC's IP address → set port 50002.

- Ensure the Robotiq URCap shows Active: Settings → System → URCaps → Robotiq Gripper → Status = Active.

- Put the robot in Remote Control mode: tap the hamburger menu (top-right) → Remote Control → Enable.

- Press Play (▶) to run the ExternalControl program — the robot is now waiting for ROS2 to connect.

The teach pendant screen should show 'External Control - Running' and a blinking connection indicator. If it shows 'Waiting for connection' that is also normal at this stage.

## 2.3  PC / ROS2 Environment

- Verify the PC and UR12e are on the same network subnet.

- Ping the robot from the PC terminal:

ping <robot_ip>

- Confirm the workspace is built and sourced:

cd /ros2_ws && colcon build --packages-select my_ur_description

source /ros2_ws/install/setup.bash

# 3  Startup Procedure (Step-by-Step)

You will need 3 separate terminals. Open them all before starting.

## 3.1  Terminal 1 — Launch Robot Stack

This is the main launch: ros2_control, MoveIt2, RViz, and the UR driver all start together.

cd /ros2_ws

source install/setup.bash

ros2 launch my_ur_description my_robot.launch.py \

use_fake_hardware:=false \

robot_ip:=<YOUR_ROBOT_IP>

Replace <YOUR_ROBOT_IP> with the actual IP shown on the teach pendant (Settings → System → Network). Example: 192.168.1.100

Expected startup sequence in the terminal:

- [INFO] Loading hardware 'URHardwareInterface'

- [INFO] Successful initialization of hardware 'URHardwareInterface'

- [INFO] Loading hardware 'RobotiqGripperHardwareInterface'

- [INFO] Starting primary client pipeline

- [INFO] Activating HW interface

- [INFO] You can start planning now!  ← Wait for this line

⏱  This takes approximately 15–30 seconds. Do NOT proceed to Terminal 2 until you see 'You can start planning now!'

## 3.2  Terminal 2 — Launch Real Gripper Server

This starts the robotiq_real_gripper_server.py bridge which handles all URScript communication with the physical 2F-85.

cd /ros2_ws

source install/setup.bash

ros2 run my_ur_description robotiq_real_gripper_server.py \

--ros-args -p robot_ip:=<YOUR_ROBOT_IP>

Expected output:

- [INFO] [robotiq_real_gripper_server]: Connecting to gripper...

- [INFO] [robotiq_real_gripper_server]: Activating gripper (startup, once only)...

- [INFO] [robotiq_real_gripper_server]: Gripper activation complete.

- [INFO] [robotiq_real_gripper_server]: Gripper server ready. Listening on /robotiq_bridge/command

Activation runs once at startup — this eliminates the ~1–2 second delay that was present in earlier versions. Subsequent gripper moves are immediate.

## 3.3  Terminal 3 — Verify System Health

Use this terminal to confirm everything is running correctly before executing any motion.

Check controllers are active:

ros2 control list_controllers

Expected output:

joint_state_broadcaster  [joint_state_broadcaster/JointStateBroadcaster]  active

scaled_joint_trajectory_controller  [ur_controllers/ScaledJointTrajectoryController]  active

robotiq_gripper_controller  [joint_trajectory_controller/JointTrajectoryController]  active

Check joint states are publishing:

ros2 topic echo /joint_states --once

Check gripper bridge is live:

ros2 topic echo /robotiq_bridge/position

If any controller shows 'inactive' instead of 'active', check for errors in Terminal 1 and verify the robot is in Remote Control mode on the teach pendant.

# 4  Running Motion Scripts

## 4.1  API Reference (UR12eController)

All motion scripts use the UR12eController class from test_move_xyz_theta_noflip_gmove.py. The gripper now communicates via /robotiq_bridge/command instead of directly to the action server.

| Method | Parameters | Description |
| --- | --- | --- |
| bot.jmove([j1,j2,j3,j4,j5,j6]) | 6 joint angles (rad) | Joint-space move — fastest, most predictable |
| bot.move_xyz_no_flip(x,y,z) | metres, base_link frame | Cartesian move, gripper pointing down, no wrist flip |
| bot.move_xyz_theta_no_flip(x,y,z,θ) | metres + radians | Cartesian move with gripper rotation θ around Z-axis |
| bot.move_pose_no_flip(x,y,z,qx,qy,qz,qw) | metres + quaternion | Full 6-DOF pose control with IK seeded from home |
| bot.gripper_move(pos) | 0.0–0.8 rad | 0.0 = fully open, 0.8 = fully closed |
| bot.check_grasp_success() | — | Returns True if gripper position < 0.75 (object detected) |

## 4.2  Coordinate System

| Axis | Direction (base_link frame) |
| --- | --- |
| X + | Forward — away from robot base flange, toward workspace |
| Y + | Left — when standing in front of the robot facing it |
| Z + | Up — above the workcell |
| Home | [-1.5707, -2.3562, 2.3562, -1.5707, -1.5707, 0.0] (shoulder at -90°, elbow raised) |

## 4.3  Running Your First Motion

Use Terminal 3 (or open a fourth terminal). Always source the workspace first.

cd /ros2_ws && source install/setup.bash

python3 src/my_ur_description/scripts/test_move_xyz_theta_noflip_gmove.py

First run safety: The script executes at 10% velocity scaling (max_velocity_scaling_factor=0.1). Watch the robot carefully. Keep your hand near the E-Stop for the first several executions.

## 4.4  Example Pick-and-Place Snippet

The following shows the standard pattern for a gripper-based pick cycle:

# 1. Go to safe home joint position bot.jmove([-1.5707, -2.3562, 2.3562, -1.5707, -1.5707, 0.0]) # 2. Open gripper before approach bot.gripper_move(0.0) # Float32 → /robotiq_bridge/command # 3. Move above pick location bot.move_xyz_no_flip(0.5, -0.2, 0.4) # approach height # 4. Lower to grasp height bot.move_xyz_no_flip(0.5, -0.2, 0.18) # object height + gripper offset # 5. Close gripper bot.gripper_move(0.8) # close — real 2F-85 will actuate # 6. Verify grasp if bot.check_grasp_success(): bot.move_xyz_no_flip(0.5, -0.2, 0.4) # lift bot.move_xyz_no_flip(0.5, 0.2, 0.4) # carry to place bot.move_xyz_no_flip(0.5, 0.2, 0.18) # lower to place bot.gripper_move(0.0) # release

# 5  Kinematic Calibration (One-Time Setup)

Each physical UR robot has unique kinematic parameters. Calibration corrects sub-millimetre deviations from the nominal model and improves absolute accuracy.

## 5.1  Extract Calibration from the Robot

# Install calibration tool (if not already installed)

sudo apt install ros-humble-ur-calibration

# Extract from your specific robot

ros2 launch ur_calibration calibration_correction.launch.py \

robot_ip:=<YOUR_ROBOT_IP> \

target_filename:="${HOME}/my_ur12e_calibration.yaml"

# Copy into the package

mkdir -p /ros2_ws/src/my_ur_description/config/calibration

cp ~/my_ur12e_calibration.yaml \

/ros2_ws/src/my_ur_description/config/calibration/

## 5.2  Use Calibration in Launch

Pass the calibration file when launching (replace the standard launch command from Section 3.1):

ros2 launch my_ur_description my_robot.launch.py \

use_fake_hardware:=false \

robot_ip:=<YOUR_ROBOT_IP> \

kinematics_parameters_file:=/ros2_ws/src/my_ur_description/config/calibration/my_ur12e_calibration.yaml

Without calibration, Cartesian positioning accuracy is approximately ±3–5mm. With calibration it improves to ±0.1mm. For pick-and-place with the 2F-85 this makes a significant difference in grasp reliability.

# 6  Troubleshooting

## 6.1  Robot / Connection Issues

| Symptom | Resolution |
| --- | --- |
| 'Could not connect to robot' on launch | 1. Verify robot_ip with: ping <ip> 2. Check UR12e is in Remote Control mode 3. Confirm ExternalControl program is running on pendant |
| 'Protective stop' on pendant | 1. Investigate cause (collision? joint limit?) 2. Clear on pendant: Robot tab → Unlock 3. Re-run ExternalControl program |
| Calibration mismatch warning | Extract calibration file per Section 5 and pass kinematics_parameters_file argument. This warning does not block operation. |
| 'scaled_joint_trajectory_controller inactive' | Robot likely not in Remote Control. Check pendant, re-run ExternalControl program, then restart ROS2 launch. |

## 6.2  Gripper Issues

| Symptom | Resolution |
| --- | --- |
| Gripper does not move when commanded | 1. Confirm gripper server is running (Terminal 2) 2. Check: ros2 topic echo /robotiq_bridge/command 3. Verify Robotiq URCap is Active on pendant 4. Repower gripper: turn off UR controller, wait 10s, turn on |
| Gripper moves but RViz does not update | The robotiq_gripper_controller mirror step may have failed. Check Terminal 2 for errors. Ensure no second instance of any server is running. |
| 'Grasp Failed: Gripper is empty' on every pick | Normal if no object is present. If object is present: check Z-coordinate (lower grasp height), check gripper force setting, inspect finger alignment. |
| Double-spawn warnings (solved in v1.1.10) | Verify you are on the v1.1.10_gripper_action_server_bug_cleared tag. Do NOT run the old bridge scripts alongside the new server. |

## 6.3  Motion Planning Issues

| Symptom | Resolution |
| --- | --- |
| 'Planning failed' or 'IK failed' | 1. Target position may be outside workspace 2. Try a different seed: seed_joints=home 3. Increase allowed_planning_time (currently 5.0s) 4. Check for collision objects in planning scene |
| Wrist 'spinning 270°' instead of 90° | Use move_xyz_theta_no_flip() instead of move_xyz_theta(). The no_flip variant normalises wrist_3_joint to [-π, π]. |
| Robot moves to wrong orientation | Default 'pointing down' quaternion is [1,0,0,0]. Adjust and re-test. Use move_xyz_theta_no_flip() with θ=0 for straight down. |

## 6.4  Health Check Commands

# Controller status

ros2 control list_controllers

# Topic publishing rates

ros2 topic hz /joint_states

ros2 topic hz /robotiq_bridge/position

# Action server availability

ros2 action list

# Live joint state

ros2 topic echo /joint_states --once

# 7  Shutdown Procedure

## 7.1  Normal Shutdown

- Move the robot to a safe home position before shutting down:

# In a Python script or terminal:

bot.jmove([-1.5707, -2.3562, 2.3562, -1.5707, -1.5707, 0.0])

- Stop Terminal 3 (motion scripts): Ctrl+C

- Stop Terminal 2 (gripper server): Ctrl+C  — wait for clean shutdown message

- Stop Terminal 1 (robot launch): Ctrl+C  — wait for all nodes to terminate

- On the teach pendant: stop the ExternalControl program → tap the Stop (■) button

- Power down the UR12e controller using the power button on the control cabinet

## 7.2  Emergency Stop Procedure

In any unsafe situation: press the E-Stop immediately. The robot will brake and enter protective stop mode. Do NOT attempt to restart until the cause is fully understood.

- Press E-Stop button.

- Ensure all people are clear of the robot.

- Investigate the cause — collision? Unexpected motion? Software error?

- Reset E-Stop (twist to unlock) only when the cause is resolved.

- On pendant: clear the protective stop → Unlock.

- Perform a slow, manual jog with the teach pendant to verify the robot is undamaged.

- Only then restart the ROS2 stack if resuming automated operation.

## 7.3  Safety Reminders

- Always operate at reduced speed (≤25%) until you have verified all motion targets are correct in simulation first.

- Test every new motion script in URSim (use_fake_hardware:=false, robot_ip:=127.0.0.1) before running on the real robot.

- Keep at least one person near the E-Stop at all times during automated operation.

- The 2F-85 generates 235 N of grip force at maximum — keep fingers and cables away from the jaw area.

- Never modify safety stop or collision sensitivity settings without a proper risk assessment.

# 8  Quick Reference Card

## 8.1  Full System Startup (copy-paste ready)

Set ROBOT_IP once, then paste each terminal block.

Terminal 1 — Robot stack:

export ROBOT_IP=192.168.1.100   # ← change to your robot's IP

cd /ros2_ws && source install/setup.bash

ros2 launch my_ur_description my_robot.launch.py \

use_fake_hardware:=false robot_ip:=${ROBOT_IP}

# Wait for: 'You can start planning now!'

Terminal 2 — Gripper server:

export ROBOT_IP=192.168.1.100

cd /ros2_ws && source install/setup.bash

ros2 run my_ur_description robotiq_real_gripper_server.py \

--ros-args -p robot_ip:=${ROBOT_IP}

# Wait for: 'Gripper server ready. Listening on /robotiq_bridge/command'

Terminal 3 — Verify then run:

cd /ros2_ws && source install/setup.bash

ros2 control list_controllers   # confirm all 3 are 'active'

python3 src/my_ur_description/scripts/test_move_xyz_theta_noflip_gmove.py

## 8.2  Version Info

| Item | Value |
| --- | --- |
| Git branch | ur12e_2f-85_action_server |
| Git tag | v1.1.10_gripper_action_server_bug_cleared |
| ROS2 | Humble Hawksbill |
| Workspace | /ros2_ws/src/my_ur_description |
| Gripper range | 0.0 rad (open) → 0.8 rad (closed) / 0–85 mm |
| Gripper command topic | /robotiq_bridge/command (Float32) |
| Gripper feedback topic | /robotiq_bridge/position (Float32) |
| Velocity scaling | 10% default (max_velocity_scaling_factor=0.1) |
| Home joints (rad) | [-1.5707, -2.3562, 2.3562, -1.5707, -1.5707, 0.0] |